Detailed construction directions:

- Print out all templates (actual size) and these directions.

- Cut out all the pieces from ½ inch pine or cedar.  You can use ¾ inch material but ½ inch is strongly preferred.

- Locate the center of the opening in the front wall (3 – 5/8 inches up from the floor and centered left-to-right) then drill the 7/8 inch hole.

- If you plan to add a perch, drill a ¼ inch hole 1 – 3/4 inches up from the floor and centered left-to-right then glue in a 2 inch long dowel.

- Sand all pieces before assembly.

- Position, glue, and nail the left wall (C) to the rear wall.  Glue the cut edge of C to the flat side of the rear wall while ensuring the bottom edges remain flush. (See the Wall Assembly diagram for correct placement.) *****NOTE *****There will be a small gap between the side walls and the roof when the house is finished.

- Position, glue, and nail the left wall (D) to the rear wall.  Glue the cut edge of D to the flat side of the rear wall while ensuring the bottom edges remain flush.

- Position, glue, and nail the front wall onto the rest.  Again, glue the cut edges of the side walls while ensuring the bottom edges remain flush.

- Position, glue, and nail the roof A to roof B.  Be sure to glue the cut edges of roof A to the flat side of roof B as indicated in the original plan drawing.

- Drill the six screw holes in the base and set them with a countersink. If you do not have a countersink bit, carefully use a larger bit to make the bevel. To properly locate the holes, set the wall assembly on the base with the rear wall and side walls flush to the rear and side edges of the base then trace the walls lightly with a pencil. Locate the holes in the center of the wall thickness.

- Position, glue, and nail the roof onto the wall assembly.  Use a square to keep the walls true.

- To prolong the life of the house, it is best, but not essential, to cover the roof with a roofing paper and light weight shingles.

- Attach the bottom with six screws to allow you to clean the house each fall.  Also, wax the screws before inserting them to make removal easier.

- Add two medium sized eyehooks to the peak of the roof to facilitate hanging your birdhouse. A heavy-duty steel fishing leader works well for the hanging wire.

- You can either leave the wood natural or paint it for a longer life.  However, do NOT finish the inside of the house.

- Hang the house six to ten feet above the ground and in a location that is not ‘baked’ by the summer sun.

- Enjoy your renters when they move in.