Detailed construction directions:

- Print out all templates (actual size) and these directions.

- Cut out all the pieces from ½ inch pine or cedar.  You can use ¾ inch material but ½ inch is strongly preferred. The use of ¾ inch material may force you to change some dimensions.

- Locate the center of the opening in the front wall (3 – 5/8 inches up from the bottom edge and centered left-to-right) then drill the 7/8 inch hole.

- If you plan to add a perch, drill a ¼ inch hole 2 – 1/4 inches up from the bottom edge and centered left-to-right then glue in a 2 inch long dowel.

- Sand all pieces before assembly.

- Cut the mitered angles on the floor and roof pieces at 9 degrees. (10 degrees is close enough.)

- Position, glue, and nail the floor to the rear and front walls as shown in the original drawing.

- Position, glue, and nail the floor and walls to the right wall as shown in the original drawing. Center this assembly on the wall and maintain a 3½ inch opening at the top of the walls.

- Position, glue, and nail the roof pieces to the right wall as shown in the original drawing. .)

*****NOTE *****There will be a small gap between the walls and the roof when the house is finished.

- Locate and drill five screw holes in the left wall and set them with a countersink. If you do not have a countersink bit, carefully use a larger bit to make the bevel. To properly locate the holes, set the right wall on the assembly then trace the walls lightly with a pencil. Locate the holes in the center of the wall thickness.

- To prolong the life of the house, it is best, but not essential, to cover the roof with a roofing paper and light weight shingles.

- Attach the left side with five screws to allow you to clean the house each fall.  Also, wax the screws before inserting them to make removal easier.

- Add two medium sized eyehooks to the peak of the roof to facilitate hanging your birdhouse. A heavy-duty steel fishing leader works well for the hanging wire.

- You can either leave the wood natural or paint it for a longer life.  However, do NOT finish the inside of the house.

- Hang the house six to ten feet above the ground and in a location that is not ‘baked’ by the summer sun.

- Enjoy your renters when they move in.