Detailed construction directions:

- Print out all templates (actual size) and these directions.

- Cut out all the pieces from ½ inch pine or cedar.  You can use ¾ inch material but ½ inch is strongly preferred. The use of ¾ inch material may force you to change some dimensions.

- Locate the center of the opening in the front wall (3 – 1/4 inches up from the bottom edge and centered left-to-right) then drill the 7/8 inch hole.

- Locate and drill the a ¼ inch hole for the upper perch in each side piece as indicated on the original drawing.

- If you plan to add a perch, drill a ¼ inch hole 2 inches up from the bottom edge and centered left-to-right then glue in a 2 inch long dowel.

- Locate and drill the two screw holes in the front and back walls and set them with a countersink. If you do not have a countersink bit, carefully use a larger bit to make the bevel. Locate the holes in the center of the floor thickness.

- Sand all pieces before assembly.

- Cut the mitered angles on roof pieces at 22 degrees.

- Position, glue, and nail the box top to the rear and front walls as shown in the original drawing.

- Position the floor within the walls but do NOT glue or nail it.  Place a thin piece of plastic or cardboard between the floor and the back wall to keep a tiny space to allow removal of the floor after assembly. Tape the floor to the wall assembly.

- Position, glue, and nail the wall assembly to the left wall as shown in the original drawing. Do NOT glue the floor in.

- Position, glue, and nail the roof pieces to the left wall as shown in the original drawing. .)

*****NOTE *****There will be a small gap between the walls and the roof when the house is finished.

- To prolong the life of the house, it is best, but not essential, to cover the roof with a roofing paper and light weight shingles.

- Position, glue, and nail the right side to the assembly taking car to not glue the floor.

- Remove the floor. You may need to tap in at an angle to remove it.

- If the floor did not slide out easily, sand the edges until you have a very a loose fit.

- Use four screws to install the house and allow you to clean it each fall.  Also, wax the screws before inserting them to make removal easier.

- Add two medium sized eyehooks to the peak of the roof to facilitate hanging your birdhouse. A heavy-duty steel fishing leader works well for the hanging wire.

- You can either leave the wood natural or paint it for a longer life.  However, do NOT finish the inside of the house.

- Hang the house six to ten feet above the ground and in a location that is not ‘baked’ by the summer sun.

- Enjoy your renters when they move in.